BASELINE DEFINITION for EPC-19-060

October, 2020

Recipient Project Manager: Sarah Kurtz

Commission Agreement Manager: Jeffrey Sunquist

Wind

The RESOLVE price inputs for wind are highly dependent on the location of the plant with the lowest prices found for Greater_Imperial_Wind and the highest for Wyoming_Wind.

Table RESOLVE Wind RSP inputs

|  | Greater Imperial |  | Humboldt |  | Wyoming |  |
| --- | --- | --- | --- | --- | --- | --- |
| Year | Annualized Cost ($/KW) | Fixed O&M ($/KW) | Annualized Cost ($/KW) | Fixed O&M ($/KW) | Annualized Cost ($/KW) | Fixed O&M ($/KW) |
| 2020 | 71.92 | 45.11 | 84.00 | 42.44 | 194.67 | 49.64 |
| 2022 | 89.35 | 44.47 | 99.69 | 41.85 | 217.47 | 48.91 |
| 2026 | 111.69 | 43.17 | 120.81 | 40.64 | 243.91 | 47.44 |
| 2030 | 112.08 | 41.89 | 121.52 | 39.47 | 242.14 | 45.96 |
| 2045 | 115.44 | 36.95 | 125.71 | 34.87 | 240.18 | 40.45 |

Table Descriptions of RESOLVE 2019-2020 – Oct. 4, 2019

|  | Greater Imperial |  | Humboldt |  | Wyoming |  |
| --- | --- | --- | --- | --- | --- | --- |
| Year | Capital Upfront Cost (2016$/KW) | LCOE (2016$/ MWh) | Capital Upfront Cost (2016$/KW) | LCOE (2016$/ MWh) | Capital Upfront Cost (2016$/KW) | LCOE (2016$/ MWh) |
| 2020 | 1,615 | 39 | 1,624 | 42 | 1,476 | 27 |
| 2022 | 1,602 | 45 | 1,612 | 47 | 1,443 | 32 |
| 2026 | 1,580 | 52 | 1,593 | 54 | 1,387 | 39 |
| 2030 | 1,563 | 52 | 1,580 | 54 | 1,340 | 38 |

Table 2020 ATB Wind cost inputs

|  | Class 6 moderate |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Year | Capital Upfront Cost (2018$/KW) | Fixed O&M (2018$/KW) | LCOE (2018$/ MWh) |  |  |  |
| 2020 | 1,611 | 42 | 37 |  |  |  |
| 2022 | 1,579 | 42 | 35 |  |  |  |
| 2026 | 1,503 | 40 | 32 |  |  |  |
| 2030 | 1,412 | 39 | 30 |  |  |  |
| 2045 | 1,155 | 35 | 24 |  |  |  |

Table 18 in Prelim_Results_Proposed_Inputs_and_Assumptions_2019-2020_10-4-19

Notes that Baseline wind in CAISO is 8.549 GW in 2020 and 8.649 GW in 2022 and beyond. Table 19 gives information about other zones.  Table 26 summarizes the assessments of available expansion.

Geothermal

RESOLVE has already some values for the inputs in the 3 and they mentioned that they were using the NREL ATB as their base. But the NREL ATB costs are decreasing gradually year-to-year whereas the RESOLVE inputs are increasing. The costs are listed here:

|  |  | NREL ATB |  |  | RESOLVE RSP inputs |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Annualized Cost* ($/KW) | Fixed O&M ($/KWh) | Total Annual Cost ($/KW) | Fixed Annual Cost ($/KW) | Fixed O&M ($/KWh) | Total Annual Cost ($/KW) |
| 2020 | 280.18 | 136 | 416.18 | 477 | 149 | 626 |
| 2022 | 272.87 | 134 | 406.87 | 479 | 149 | 628 |
| 2026 | 258.25 | 130 | 388.25 | 542 | 149 | 691 |
| 2030 | 246.74 | 127 | 373.74 | 561 | 149 | 710 |
| 2045 | 225.95 | 127 | 352.95 | 555 | 149 | 704 |

*Assumptions for the NREL ATB Calculations: Lifetime=30 years & Discount rate=4.8%

| Two new plants with their PPAs |  |  |
| --- | --- | --- |
| PPA rate ($/kWh) | Annual (kWh/kW) | Annual Revenue ($/kW) |
| 0.074 | 7884 | 583 |
| 0.068 | 7008 | 477 |
| Capacity Factor | 90% for the first PPA |  |
|  | 80% for the second PPA |  |

If we assume the RESOLVE inputs as our costs, then the annual revenue becomes smaller than the cost, which does not seem feasible enough. Therefore, it would be helpful if we can understand why RESOLVE has taken these values as the cost.

Again, from the recent “SB100 report” and “Inputs & Assumptions:2019-2020 Integrated Resource Planning”, there is a list of the implied LCOE for 5 geothermal resources:

|  | Inputs & Assumptions: 2019-2020 Integrated Resource Planning |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Resources | Capacity Factor | 2020 | 2022 | 2026 | 2030 |
| Greater_Imperial_Geothermal | 88% | $81 | $81 | $89 | $92 |
| Inyokern_North_Kramer_Geothermal | 80% | $97 | $98 | $108 | $111 |
| Northern_California_Ex_Geothermal | 81% | $91 | $91 | $100 | $103 |
| Riverside_Palm_Springs_Geothermal | 80% | $88 | $88 | $97 | $100 |
| Solano_Geothermal | 90% | $78 | $78 | $86 | $88 |

This report was published on February 2020 and you can see that there is an increase in the LCOE from 2020 to 2030.

|  | Inputs & Assumptions: CEC SB100 Joint Agency Report |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Resources | Capacity Factor | 2027 | 2030 | 2035 | 2040 | 2045 |
| Greater_Imperial_Geothermal | 88% | $79 | $79 | $78 | $77 | $76 |
| Inyokern_North_Kramer_Geothermal | 80% | $87 | $87 | $86 | $85 | $83 |
| Northern_California_Ex_Geothermal | 81% | $86 | $85 | $85 | $84 | $82 |
| Riverside_Palm_Springs_Geothermal | 80% | $87 | $87 | $86 | $85 | $83 |
| Solano_Geothermal | 90% | $77 | $77 | $76 | $75 | $74 |

This was published on June 2020, which indicates that the LCOE should be increased from 2027 to 2045. In addition, if we check the value for 2030 in both reports, we find the numbers different to each other but those should be same.

The capacity limits for new builds of geothermal are small compared with the amount that would be needed for geothermal to make a big difference. Specifically, the RSP limits the Geothermal growth to total about 2 GW, resulting in a total of about 3 GW. If this amount could be doubled or tripled by 2045, it could make a significant difference in the value of long-duration storage.

| Resource | Capacity limit (MW) | Cumulative new build in 2045 for RSP (MW) | Note |
| --- | --- | --- | --- |
| BANC_Geothermal_for_Other |  |  | No new builds |
| CAISO_Geothermal_for_Other |  |  | No new builds |
| IID_Geothermal_for_Other |  |  | No new builds |
| LDWP_Geothermal_for_Other |  |  | No new builds |
| NW_Geothermal_for_Other |  |  | No new builds |
| SW_Geothermal_for_Other |  |  | No new builds |
| CAISO_Geothermal_for_CAISO |  |  | No new builds |
| IID_Geothermal_for_CAISO |  |  | No new builds |
| NW_Geothermal_for_CAISO |  |  | No new builds |
| Greater_Imperial_Geothermal | 1352.1 | 1352.1 |  |
| Inyokern_North_Kramer_Geothermal | 24 | 0 | Why didn’t it build? |
| Northern_California_Ex_Geothermal | 469 | 469 |  |
| Pacific_Northwest_Geothermal |  |  | No new builds |
| Riverside_Palm_Springs_Geothermal | 32 | 32 |  |
| Solano_Geothermal | 135 | 135 |  |
| Southern_Nevada_Geothermal | 320 | 320 |  |

Jeremiah, Could you take a look at the question of whether we should allow more geothermal to be built? I don’t know of a reference that will answer this directly, so you will need to hunt… Sarah

Financial details

Every financial calculation needs to keep track of key financial parameters.  Some of these factors are much smaller than the uncertainty in the projected costs. Nevertheless, when comparing models for differences, these can add up and direct comparison is facilitated by understand where each factor is accounted for. The table summarizes how each is handled in the two models.

Table financial details

| Financial element | RESOLVE | SWITCH | Comment |
| --- | --- | --- | --- |
| Interest rate | Technology specific – included in calculating the annualized cost from the overnight CapEx taken from NREL ATB or other source |  |  |
| Inflation rate | 2.5%/y – taken from NREL ATB and used as an adjustment to the annualized costs. While the NREL ATB provides numbers in constant 2018 dollars, the RESOLVE inputs for 2030 are in 2030 dollars, for example |  |  |
| Discount rate | Discount rate (4.8%/y) is defined in a file called period_discount_factors and is applied to all costs in the period when an expenditure is made. |  |  |
| Incentive programs | Adjusts for ITC in the annualized costs. |  |  |

| period | discount_factor | years_in_period |  |  |
| --- | --- | --- | --- | --- |
| 2020 | 1 | 1 |  |  |
| 2021 | 0.95238095 | 1 |  |  |
| 2022 | 0.90702948 | 1 |  |  |
| 2023 | 0.8638376 | 1 |  |  |
| 2024 | 1.21446556 | 1.5 |  |  |
| 2026 | 2.17056139 | 3 |  |  |
| 2030 | 4.97063153 | 9.5 |  |  |
| 2045 | 6.69516331 | 28 |  |  |

Pro Forma Financial Model (SB100 Joint Agency Report)

TN234532 “This section describes the purpose of and methodology behind the pro forma financial model. The pro forma model is a discounted cash flow model used to calculate the levelized costs of different candidate resources. The primary outputs from the model are the levelized fixed costs for each resource. Levelized fixed costs calculated by the pro forma include the overnight capital cost for each resource, financing costs (including investor returns on a project), fixed O&M costs, and any capital-based tax credits, such as the Investment Tax Credit (ITC) and the Production Tax Credit (PTC), which are used to offset capital costs.

The pro forma used for the CEC SB100 analysis assumes financing is provided by an Independent Power Producer (IPP), which reflects current development practices in which most new resources in California are third-party owned and contracted with LSEs rather than financed by LSEs themselves. Financing assumptions assumed in the pro forma model are based on NREL’s 2019 Annual Technology Baseline. (Financing assumptions include WACC, cost of debt and debt fraction. E3 adjusted NREL’s cost of debt to reflect the current rate environment. based on the spread to the Industrial Baa bond rate, as used by EIA in the Annual Energy Outlook.)

Levelized costs are calculated in the pro forma using real levelization to yield costs that are flat in real dollar terms. This approach discounts annual project costs using a nominal discount rate (nominal return on equity) and discounts energy and capacity using a real discount rate (real return on equity). This is a standard approach that yields levelized costs in flat real terms for input to the RESOLVE model.

The pro forma also requires information on variable costs (such as fuel and variable O&M) and resource performance characteristics. These inputs are considered in the pro forma financing optimization but have minimal impacts on levelized fixed costs. In addition, variable costs included in the pro forma model do not directly flow through to RESOLVE as inputs in the modeling process.”

In determining financial rates, consider https://www.nrel.gov/docs/fy20osti/76814.pdf

Summary of proposed SWITCH inputs

Comparison of differences between RESOLVE and SWITCH

References

TN234532_20200831T081744 “Input & Assumptions - CEC SB100 Joint Agency Report” June 2020