# Technology Projection 2. Renewable Energy Technologies

1. NREL Annual Technology Baseline 2020

NREL Annual Technology Baseline 2020 (ATB) projects CAPEX, fixed O&M costs, and variable O&M costs of renewable energy technologies. Table 1 summarized selected renewable energy technologies from ATB for this document. Other renewable technologies in ATB are Commercial PV, residential PV, CSP, hydropower, and offshore wind. ATB sets three projection scenarios to reflect the speed of technological innovation: Conservative, Moderate, and Advanced Scenarios. Table 2 qualitatively summarized these scenarios.

Table 1: Selected Renewable Energy Technologies in ATB

| Technology | Technology details |
| --- | --- |
| Land-based Wind | Wind Speed Class 1-10 with different CF, overnight costs |
| Utility PV | 5 representative cities (Chicago, Daggett, Kansas City, LA, Seattle) with different CF, overnight costs, and fixed O&M costs |
| Geothermal | DeepEGSBinary, DeepEGSFlash, HydroBinary, HydroFlash, NFEGSBinary, NFEGSFlash with different CF, overnight costs, and fixed O&M costs |
| Biopower | CofireNew, CofireOld, Dedicated with different overnight costs, fixed O&M costs, and variable O&M costs |

Table 2: Three Projection Scenarios

| Scenario | Description on technological innovation |
| --- | --- |
| Conservative | - Today's technology with little innovation - Continued industrial learning - Decreased public and private R&D |
| Moderate | - Widespread adoption of today's cutting edge - Expected level of innovation - Current levels of public and private R&D |
| Advanced | - Markets success of currently unproven innovation - New technology architectures - Increased public and private R&D |

ATB projects the capital costs, fixed O&M costs, and variable O&M costs. Except for biopower, renewable energy technologies have $0 variable O&M costs. All costs are given in 2018 USD. Any of these numbers do not include policy support such as subsidy.

2. Land-based Wind

ATB projects the capacity factor and overnight costs of 10 wind classes. Wind classes are defined the average wind speed as summarized in Table 3. The capacity factor and overnight costs of 10 wind classes are summarized in Figure 1 and 2, respectively.

Table 3: Wind classes in ATB

| Wind Classes | Wind Speed Range (m/s) | Weighted Average Wind Speed (m/s) | Potential Wind Plant Capacity (GW) |
| --- | --- | --- | --- |
| Class1 | > 9.0 | 9.5 | 153 |  |
| Class2 | 8.8 - 9.0 | 8.9 | 152 |  |
| Class3 | 8.6 - 8.8 | 8.7 | 304 |  |
| Class4 | 8.4 - 8.6 | 8.5 | 606 |  |
| Class5 | 8.1 - 8.4 | 8.2 | 1,222 |  |
| Class6 | 7.6 - 8.1 | 7.8 | 2,404 |  |
| Class7 | 7.1 - 7.6 | 7.4 | 2,444 |  |
| Class8 | 6.5 - 7.1 | 6.8 | 2,441 |  |
| Class9 | 5.9 - 6.5 | 6.2 | 2,406 |  |
| Class10 | 0 - 5.9 | 5.2 | 3,044 |  |
| Total |  |  | 15,176 |  |
| Wind speeds presented in table are assumed to be 110 meters above the surface |  |  |  |  |

With technological advancement, the capacity factors of all wind classes improve under moderate and advanced scenarios. Most of the improvements occurs between 2018 and 2030. Under moderate scenarios, capacity factors improve by 4 % (class 1) - 17 % (class 9 and 10).

Figure 1: Projections of capacity factor of 10 wind classes under three scenarios from ATB

The overnight capacity costs of 10 wind classes decrease under all three scenarios. Wind class 1 through 5 (8.1-8.4 m/s) have very similar cost projections. Under the moderate scenario, the overnight costs decline from around $1600/kW in 2020 to $1000/kW. Then, the lower the wind speed gets, the higher the overnight costs are. Cost reduction under the moderate scenarios are 34 % - 39 %.

Figure 2: Projections of overnight capital costs of land-based wind power in real $2018 of 10 wind classes under three scenarios from ATB

On the other hand, the fixed O&M costs are the same across 10 wind classes as shown in Figure 3. Under the moderate scenario, the fixed O&M costs linearly decrease from $42.5/kW-yr in 2020 to $33.1/kW-yr in 2050.

Figure 3: Projections of fixed O&M costs of land-based wind power in real $2018 under three scenarios from ATB

Figure 4: Comparison of ATB Projections of fixed O&M costs of land-based wind power in real $2018 with SWITCH and RESOLVE

3. Utility PV

ATB projects the capacity factor of 5 representative cities as summarized in Figure 5. The capacity factors of all cities improve under all scenarios, with technological innovation. Under moderate scenarios, capacity factors linearly improve by 11-17 % from 2020 to 2050.

Figure 5: Projections of capacity factor of 5 representative cities under three scenarios from ATB

Overnight costs and fixed O&M costs projections are identical for all representative cities. Both costs decline by 49 % from 2020 to 2050.

Figure 6: Projections of overnight costs (left) and fixed O&M costs (right) of utility PV in real $2018 under three scenarios from ATB

Figure 7: Comparison of ATB Projections of fixed O&M costs of utility PV in real $2018 with SWITCH and RESOLVE

4. Geothermal

| Table 3: Geothermal Resource and Cost Characteristics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Flash | Binary |  |  |
|  | Temp (°C) | >=200C | 150-200 | 135-150 | <135 |
| Hydrothermal | # identified sites | 21 | 22 | 17 | 59 |
|  | Total capacity (MW) | 15,338 | 2,991 | 820 | 4,759 |
|  | Avg OCC ($/kW) | 4,175 | 8,829 | 9,476 | 17,757 |
|  | Min OCC ($/kW) | 3,000 | 4,397 | 7,444 | 11,884 |
|  | Max OCC ($/kW) | 5,971 | 38,720 | 11,781 | 25,934 |
|  | Example Plant OCC ($/kW) | 4,522 | 5,870 |  |  |
| NF-EGS | # sites | 12 | 20 |  |  |
|  | Total capacity (MW) | 787 | 596 |  |  |
|  | Avg OCC ($/kW) | 11,429 | 27,330 |  |  |
|  | Min OCC ($/kW) | 9,026 | 18,974 |  |  |
|  | Max OCC ($/kW) | 18,797 | 41,694 |  |  |
|  | Example Plant OCC ($/kW) | 14,486 | 32,921 |  |  |
| Deep EGS (3-6 km) | # sites | n/a | n/a |  |  |
|  | Total capacity (MW) | 100,000+ |  |  |  |
|  | Avg OCC ($/kW) | 28,991 | 65,081 |  |  |
|  | Min OCC ($/kW) | 18,733 | 40,515 |  |  |
|  | Max OCC ($/kW) | 54,987 | 96,405 |  |  |
|  | Example Plant OCC ($/kW) | 14,486 | 32,921 |  |  |

ATB projects the overnight costs and fixed O&M costs of 6 geothermal power technologies. Table 3 summarized the resource and cost characteristics of those technologies. Figure 8 and 9 summarized the cost reduction of overnight costs and fixed O&M costs. Most of cost reduction in overnight costs occurs from 2020 to 2030. The overnight costs decline by 19 % (Hydro Binary) to 38 % (Deep EGS Flash and NF EGS Flash) from 2020 to 2050 under the moderate scenario. All of cost reduction in fixed O&M costs occurs from 2020 to 2030. The fixed O&M costs decline by 4 % (Hydro Binary) to 15 % (Deep EGS Flash) from 2020 to 2050 under the moderate scenario.

Figure 8: Projections of overnight capital costs of geothermal power in real $2018 of 6 technologies under three scenarios from ATB

Figure 9: Projections of fixed O&M costs of geothermal power in real $2018 of 6 technologies under three scenarios from ATB

5. Biopower

ATB projects three biopower technologies as shown in Table 4. Unlike other technologies, biopower cost projection have only one scenario.

Table 4: Biopower technologies in ATB

| Biopower Technologies |  |
| --- | --- |
| Dedicated | Dedicated biomass plant |
| CofireOld | Pulverized with sulfur dioxide (SO2) scrubbers and biomass co-firing |
| CofireNew | Advanced super critical with SO2 & NOx controls & biomass co-firing |

Overnight costs projection of biopower technologies are summarized in Figure 10. Overnight costs almost linearly decline by 9 % (CofireNew and CofireOld) to 14 % (Dedicated) from 2020 to 2050. Fixed and variable O&M costs are constant during the study period. Fixed O&M costs of Dedicated and others are $123/kW-yr and $40/kW-yr, respectively. Variable O&M costs of Dedicated and others are $4.7/MWh and $4.4/MWh, respectively.

Figure 10: Projections of overnight capital costs of biopower in real $2018 of 3 technologies